YRDP/YCWA (FERC no. 2246) – Study 6.2 Lower Yuba River Riparian

Request for additional analysis of hydrologic conditions for seed germination and early survival

Draft -- September 19, 2013

Note: This proposal is identical to requested additional analysis for Study 6.1 (Riparian Study on the Upper Yuba River) with the exception that stage-discharge outputs from 2-dimensional data for the lower Yuba River are to be utilized in the place of transect data.

The analysis focuses on Fremont’s cottonwood (Populus fremontii); assumes seed dispersal for Fremont’s cottonwood is from April 1 through July 15; and that the maximum stage change per day for Fremont’s cottonwood seedling germination, establishment and survival is 2.5 centimeters per day (cm/day).  Such a rate is faster than maximum root follow rates (1 to 1.5 cm/day), but potentially allows for capillary action to provide water to root tips. Actual maximum recession rate for viable seed germination may be much less (Stella et al. 2010).

- Task 1- YCWA and Relicensing Participants will jointly review average depth to discharge relationships for seven reaches of the lower Yuba River (TM 7-10 Figure 3.3-2) as a full-census substitute for transect data with stage-discharge relationship. Note that a multitude of transects have been generated from the 2-dimensional data by Dr. Pasternack, and yet the average depth to discharge relationship could be superior, for the purposes of this analysis, to any stage-discharge relationships based in transect sampling.

  - YCWA will convert the average depth to discharge relationship for each reach into an average stage to discharge relationship.

  - YCWA will provide a tool in Microsoft™ Excel, or other similar format, that shows changes in the elevation of water along various transects at different flows in comparison to average reach stage to discharge relationship.

- Task 2 – Using the tool developed in Task 1, for each of the seven reaches and for the period between April 1 and July 15 in each of the 41 years in the relicensing hydrology period of record (i.e., Water Year 1970 through Water Year 2010), YCWA will create Microsoft™ Excel tables with the following columns of output data:

- Date

- mean daily flow (cfs) With-Project Hydrology

- daily recession rate (%) With-Project Hydrology

- mean daily stage (cm) With-Project Hydrology

- daily stage change (cm) With-Project Hydrology

- mean daily flow (cfs) Without-Project Hydrology

- daily recession rate (%) Without-Project Hydrology

- mean daily stage (cm) Without-Project Hydrology

- daily stage change (cm) Without-Project Hydrology

The With-Project and Without-Project hydrology used to develop the tables will be obtained from the relicensing Water Balance/Operations Model.  YCWA will highlight table cells with a stage change of 2.5 cm or less.

YCWA and Relicensing Participants will jointly review the completed tool and tables and discuss areas of interest for limited additional analysis before proceeding to Task 4.

- Task 3 – For With- and Without-Project hydrologies, YCWA will provide for each reach in the tables developed in Task 2:

- The number of years and percent of years for which stage recession was 2.5 cm or less than 2.5 cm/day for the entire analysis period

- A summary table that has the average, maximum, and minimum by water year type of the number of consecutive days with a recession rate of 2.5 cm/day or less.

- Two frequency distributions, one based on With-Project and one based on Without-Project hydrologies, for stage increments (e.g., 3 or 5 cm) where recession did not exceed 2.5 cm/day, and no recession rate in excess of 2.5 cm/day followed in a given year. Distributions to include all years in the relicensing hydrology period of record.

In addition, YCWA will create ‘spaghetti hydrographs’ (using stage in cm) by water year type for With- and Without-Project hydrologies, and by five water year types (i.e., 10 charts per transect).  YCWA will limit y and x axis to the analysis period and will highlight line segments for which the recession rate does not preclude opportunity for germination.  Opportunity for germination in this context is defined as less than a 2.5 cma/day change and not preceding (in the same year) a day or period with daily stage change less than 2.5 cm/day.

- Task 4 – YCWA will include this analysis and results in a revised Technical Memorandum 6.2, Riparian Habitat Downstream of Englebright Reservoir.  The tables and charts developed in Tasks 2 and 3 will be included in the technical memorandum as attachments.  YCWA anticipates the revised technical memorandum will be issued within X weeks of when YCWA and Relicensing Participants collaboratively complete Task 1.

Stella, J.C.; J.J. Battles; J.R. McBride; and B.K. Orr. 2010. Riparian seedling mortality from simulated water table recession, and the design of sustainable flow regimes on regulated rivers. Restoration Ecology 18:284-294.