Study 1.2

DRAFT CHANNEL MORPHOLOGY DOWNSTREAM OF ENGLEBRIGHT DAM

June 9, 2010

[This study proposal was discussed at the 6/9 Relicensing Participants Study Proposal Development meeting.  The Forest Service requested Licensee combine this study plan with the Channel Morphology Study Plan below Englebright; Licensee said it would prefer not to since the areas and methods were so different.  The redline changes below are based on the June 9 meetings, as well as general comments regarding study plans made at previous meetings. JML 6/18]

1.0	Project Nexus

The Yuba River below Englebright Dam has an extensive history of impacts resulting from hydraulic mining activities in the Yuba River watershed.  From the late 19th century through to the early part of the 20th century, vast amounts of hydraulic mining debris and sediment was deposited in the Yuba River from the confluence of the Feather River upstream for more than 20 miles.  Sediment depths at The Narrows were more than 75 feet deep, and more than 15 feet deep near Marysville in the early 20th century (CDC 1906).  While degradation of the river bed commenced in the early 20th century with the decline in hydraulic mining activity and the construction of various debris dams, several million cubic yards of sediment remain in the Yuba River below Enblebright Dam  (James 2009).  Channel process and changes over the course of the next several decades will largely be a function of the continued degradation of the river bed through layers of mining debris.

The Englebright Dam was constructed in 1941 as a debris impoundment, and has been completely effective in blocking the downstream movement of sediment.  It is estimated that as much as 17,500 acre feet (over 28 million cu yards) of sediment is impounded behind Englebright Dam (Childs 2003).  This non-Project facility is the primary impact on sediment recruitment for the Yuba River downstream of Englebright Dam.

Given the severely altered character of the Yuba River below Englebright Dam prior to the construction of the Yuba County Water Agency’s (Licensee or YCWA) construction of the Yuba River Development Project (Project), determining impacts of the Project facility become somewhat more opaque than evaluating potential Project impacts on unaltered environments.  However, the Project has the potential to affect channel morphology due to changes in hydrologic regime caused by operation of the Project storage reservoir.

2.0	Resource Management Goals of Agencies with Jurisdiction Over the Resource to be Studied

[Agencies – Section 5.11(d)(2) states that an applicant for a new license must in its proposed study “Address any known resource management goals of the agencies or Indian tribes with jurisdiction over the resource to be studied.”  If each agency provides to YCWA a brief written description of their jurisdiction over the resource to be addressed in this study, YCWA will insert the brief description here/or attach it stating the description was provided by that agency.  If not, prior to issuing the PAD, YCWA will describe to the best of its knowledge and understanding the management goals of each agency that YCWA believes has jurisdiction over the resource addressed in this study.  Licensee] [At 5/12 meeting, agencies said they might provide a single write-up for all study proposals that covers all agencies.  Licensee said it would discus the write-up with Relicensing Participants when received, but said Licensees might add to Section 2 in some study proposals what it believed to be key information (e.g., noting USFWS ESA jurisdiction for ESA-listed fishes, and the SWRCB’s jurisdiction over water quality vis-à-vis the Basin Plan.  Licensee 5/12]

3.0	Study Goals and Objectives

The goal of the study is to quantify or characterize river form and process in the Yuba River downstream of the Englebright Dam.

The objectives of the study are to develop information necessary to meet the study goal.  Specifically, the study objectives include: 1) develop a quantitative and qualitative understanding of Project effects on substrate mobility, particle size distribution, salmonid spawning gravel distribution, and spill flow effects on channel morphology.

4.0	Existing Information and Need for Additional Information

The Yuba River downstream of Englebright Dam is one of the more thoroughly studied rivers in the Central Valley of California. A considerable amount relating to the impacts of hydraulic mining and the operation of the Yuba River as an element of the state flood control system have been developed.  Additionally, extensive information regarding A) geomorphic drivers, B) landforms and boundary conditions, C) hydrogeomorphic dynamics, D) physical habitat and ecological dynamics, and E) river management actions have been developed through time, and a compendium of the existing information specific to the Yuba River downstream of USACE’s Englebright Dam is included as Attachment 1 to this study plan.

Information is available from both previously conducted studies dating back to the early 1900s through current information from ongoing data collection, monitoring, and evaluation activities, particularly from the Yuba Accord M&E Program (M&E Program).  A summary list of recent information currently available includes, but is not limited to:

- Topographic and geologic maps, including a  digital elevation model (DEM) of the  Yuba River downstream of USACE’s Englebright Dam (M&E Program 2010)

- Hydrologic modeling and statistics for the Yuba River (YCWA 2007)

- Operations procedures for Project facilities (YCWA 2009)

- PHABSIM habitat modeling of the Yuba River conducted by Beak Consultants for the California Department of Fish and Game (Beak 1989)

- Two-dimensional hydrodynamic habitat modeling (River2D) of the Yuba River conducted by the U.S. Fish and Wildlife Service (Gard 2007; 2008)

- Two-dimensional hydrodynamic modeling (SRH-2D) of the Yuba River by U.C. Davis for the River Management Team (M&E Program 2010)

- Low-altitude aerial video of the Yuba River (YCWA 2009)

Information from ongoing data collection, monitoring, and evaluation activities, particularly from the Yuba Accord M&E Program (M&E Program) addressing geomophological conditions and physical habitat conditions in the Yuba River downstream of Englebright Dam that will be available for the conduct of this study includes, but is not limited to:

- Hydrologic water balance/operations model of the Yuba River (Relicensing Study Proposal 2.2)

- Substrate and cover classification maps of the Yuba River downstream of USACE’s Englebright Dam to characterize microhabitat and mesohabitat conditions (M&E Program).

- Mesohabitat classification map of the Yuba River (M&E Program)

Licensee believes that sufficient information generally exists to characterize the geomorpholical conditions for the Yuba River downstream of Englebright Dam without the undertaking of additional field study effort.

5.0	Study Methods and Analysis

5.1		Study Area

For the purpose of this study, the study area includes the Yuba River from the United States Army Corps of Engineers (USACE) Englebright Dam to the Feather River.

5.2		General Concepts

The following general concepts apply to the study:

- Personal safety is the most important consideration of each fieldwork team.  If YCWA determines the information cannot be collected in a safe manner, YCWA will develop other safe methods to collect the data.

- YCWA will make a good faith effort to obtain permission to access private property where needed well in advance of entering the property.

- The schedule for the study is reasonably flexible to accommodate unforeseen problems that may affect the schedule.

- Field crews may make minor modifications to the study proposal in the field to accommodate actual field conditions and unforeseen problems.  When modifications are made, YCWA field crew will follow the protocols in this study proposal.

- YCWA’s performance of the study does not presume that YCWA is responsible in whole or in part for measures that may arise from the study.

- Global Positioning System (GPS) data will be collected in a manner that meets or exceeds the United States Geological Survey’s (USGS) “National Map Accuracy Standards” for published maps.  All GPS data will be in the Universal Transverse Mercator (UTM) Coordinate System, using the North American Datum 1983 and stored in Environmental Science Research Institute (ESRI) Shapefile format.

- Field crews will be trained on and provided with materials (e.g. Quat) for decontaminating their boots, waders, and other equipment between study sites.  Major concerns are amphibian chytrid fungus, and invasive invertebrates (e.g. zebra mussels, Dreissena polymorpha).  This is of primary importance when moving: 1) between tributaries and mainstem reaches; 2) moving between basins (e.g. Middle Yuba River, North Yuba River, and Yuba River); and 3) moving between isolated wetlands or ponds and river or stream environments.

5.3		Methods

This study will consist of the following four steps: 1) compile data from previously conducted studies; 2) compile ongoing data collection and information; 3) conduct the analyses necessary to accomplish the previously stated goals and objectives; and 4) prepare report.

5.3.1		Step 1 – Compile Data from Previously Conducted Studies

Information regarding geomorphology in the Yuba River downstream of Englebright Dam will be obtained from previously conducted studies, including the dates and locations of data collection to the extent possible.

5.3.2		Step 2 – Compile Ongoing Data Collection and Information

Data collected during the Yuba Accord M&E Program will be compiled for this study plan report. Data compilation and analyses conducted by the RMT for the Yuba Accord M&E Program will be obtained and utilized from the following ongoing monitoring and evaluation activities associated with the M&E Program Protocols and Procedures:

- Flow and Water Temperature Monitoring

- Topographic Mapping (Digital Elevation Model) – physical habitat assessment

- Substrate and Cover Mapping – spawning/juvenile rearing habitat characterization

- 2-D Hydrodynamic Modeling – physical habitat dynamics and availability

- Mesohabitat Classification – physical habitat characterization

- Riparian Vegetation Mapping – juvenile rearing habitat characterization

5.3.5	Step 3 – Analyze Data

The goal of the study is to quantify or characterize river form and process.  Table 5.3-1 presents the relationship between potential channel morphology issues, data to be collected by this study, and data analysis that will occur as part of this study.

Table 5.3-1.  Relationship between perceived channel morphology issues, data to be collected by this study, and data analysis that will occur as part of this study.

| Issue | Data | Analysis |
| --- | --- | --- |
| Project effects on channel morphology and channel condition below Project facilities | - Longitudinal profile - Cross sections - Substrate - Stage-discharge relationship - Hydrologic information – regulated by Project, non-regulated by Project, and unimpaired - Age and function of riparian zone - Review of historical aerial photographs - Sediment mobility | - Longitudinal profile, cross sections, substrate will be used in the sediment transport model to show at what point the existing bed is mobile; combining with hydrology data provides the frequency of mobility under regulated and unimpaired conditions. - Stage-discharge relationship provides at what flow various surfaces in the riparian zone are inundated; combining with hydrology data provides the frequency of inundation for regulated by Project, non-regulated by Project, and unimpaired conditions. - Age and function of riparian zone provides a history of disturbance and role of riparian zone in shape and form of channel. - Assessment of channel and bank stability provides how likely the channel is to move from its current form - Historical photos show the relationship of current form and prior form (depending upon the photos available) - Sediment mobility analysis will show the flows at which the existing bed is mobile in regulated by Project, non-regulated by Project, and unimpaired conditions. |

Table 5.3-1.  (continued)

| Issue | Data | Analysis |
| --- | --- | --- |
| Project effects on floodplains | - Terrain map of river - Stage-discharge relationship - Hydrologic information - Age and function of riparian zone - Historical aerial photographs | - Terrain map of river provides the location and elevation of floodplains. - Stage-discharge relationship provides at what flow various surfaces are inundated; combining with hydrology data provides the frequency of inundation for regulated by Project, non-regulated by Project, and unimpaired conditions. - Age and function of riparian zone provides the history of floodplain development and role vegetation plays in the history, development and future. - Historical photos show the history and interaction of the active channel with floodplains, conversion to or from terraces; changes in vegetation; disturbance history. |
| Project effects on bedload distribution | - Channel armoring | - Ratio of surface to sub-surface particles - surface layer is commonly coarser than the sub-surface, and the size distribution of the sub-surface gravel is often similar to that of the transported bedload. Low values of D50surface :D50subsurface (e.g., less than 1.3 means relatively weak armoring) are generally indicative of relatively high mean annual sediment transport rates, whereas high values of D50surface : D50subsurface (e.g., greater than 4 means relatively strong armoring) are generally indicative of relatively low mean annual sediment transport rates. |
| Project effects on particle size and composition from minimum flow & uncontrolled spill | - Summary of spill history | - Discussion of channel form, sediment size and distribution as it relates to hydrology created by releases from minimum flow releases flow, and erosion and/or hydrology due to spill releases from Project dams |

5.3.6	Step 6 – Prepare Report

At the conclusion of the study, YCWA will prepare a report that includes the following sections: 1) Study Goals and Objectives; 2) Methods; 3) Results; 4) Discussion; and 5) Description of Variances from the FERC-approved study proposal, if any.  The report will include the following attachments:

- Map of river (*.PDF format) including topography, substrate, riparian vegetation, and other river features

- Floodplain inundation map, showing inundation area at 1:1, 1:2, 1:5, 1:10 and 1:20 return interval flood flows

- Summary of riparian condition.

- The hydraulic/sediment transport model input and output files.

6.0		Study-Specific Consultation

This study does not require any study-specific consultation.

7.0	Schedule

Licensee anticipates the schedule to complete the study as follows assuming the PAD is filed on November 1, 2010, and FERC issues its Study Determination by October 4, 2011:

Compile Data from Previous Studies (Step 1)	October 2011 - May 2012

Compile Data from Ongoing Studies (Step 2)	May - August 2012

Analysis (Step 3)	August - September 2012

Prepare Report (Step 4)	September - October 2012

8.0		Consistency of Methodology with Generally Accepted Scientific Practices

The methods presented in this study plan are consistent with other generally accepted scientific study methods concerning geomorphological function assessment, including those conducted by the Resource Agencies in California.

9.0	Level of Effort and Cost

[Relicensing Participants – Licensee will include a cost range estimate for this study in its Proposed Study Plan.  Licensee]

10.0	References Cited

California Debris Commission (CDC), 1906, Map of the Yuba River, California from the Narrows to its mouth in the Feather River. Made under direction of Major Wm. W. Harts, U.S. Army Corps of Engineers, by G.G. McDaniel, Jr., August to Nov., 1906: scale 1:9,600.

Childs, Jonathan R., Noah P. Snyder, and Margaret A. Hampton.  Bathymetric and geophysical surveys of Englebright Lake, Yuba-Nevada Counties, California US Geological Survey Open-File Report 03-383

James, L.A., Singer, M.B., Ghoshal, S., and Megison, M., 2009, Historical channel changes in the lower Yuba and Feather Rivers, California: Long-term effects of contrasting river-management strategies, in James, L.A., Rathburn, S.L., and Whittecar, G.R., eds., Management and Restoration of Fluvial Systems with Broad Historical Changes and Human Impacts: Geological Society of America Special Paper 451, p. 57–81, doi: 10.1130/2008.2451(04).

Yuba County Water Agency (YCWA), 2010.  Existing Information Attachment, Fluvial Geomorphology Downstream of Englebright Dam.  Prepared by Dr. Greg Pasternack for YCWA, June 2010.

Attachment 1

Lower Yuba River

Fluvial Geomorphology

Existing Information

Assessment

Englebright Dam was constructed by the California Debris Commission in 1941; is owned, operated and maintained by the United States Army Corps of Engineers; and is not included as a Project facility in FERC licenses for the Yuba River Development Project.

M&E Program documents and work products are located at the River Management Team web site, www.yubaaccordrmt.com.