Study 7.2

DRAFT POTENTIAL NARROWS 2 POWERHOUSE INTAKE EXTENSION

July 12, 2010

[This study proposal was discussed at the 6/9 Relicensing Participants Study Proposal Development meeting.  There was some discussion regarding the need for the study, but it was concluded it would be beneficial.  The redline changes below reflect comments at the 6/9 meeting and general comments made at previous meetings. JML 6/9]

1.0	Project Nexus

Yuba County Water Agency’s (Licensee or YCWA) continued operation and maintenance (O&M) of the Yuba River Development Project (Project) may affect fish in the Yuba River downstream of the United States Army Corps of Engineers’ (USACE) Englebright Dam due to water temperature.  [In their May 11 comments, agencies requested additional description of potential effects.  Licensee said it did not propose to add this but would consider wording if provided by agencies.  Licensee 5/12]

2.0	Resource Management Goals of Agencies with Jurisdiction Over the Resource to be Studied

[Agencies – Section 5.11(d)(2) states that an applicant for a new license must in its proposed study “Address any known resource management goals of the agencies or Indian tribes with jurisdiction over the resource to be studied.”  If each agency provides to YCWA a brief written description of their jurisdiction over the resource to be addressed in this study, YCWA will insert the brief description here/or attach it stating the description was provided by that agency.  If not, prior to issuing the PAD, YCWA will describe to the best of its knowledge and understanding the management goals of each agency that YCWA believes has jurisdiction over the resource addressed in this study.  Licensee] [At 5/12 meeting, agencies said they might provide a single write-up for all study proposals that covers all agencies.  Licensee said it would discus the write-up with Relicensing Participants when received, but said Licensees might add to Section 2 in some study proposals what it believed to be key information (e.g., noting USFWS ESA jurisdiction for ESA-listed fishes, and the SWRCB’s jurisdiction over water quality vis-à-vis the Basin Plan.  Licensee 5/12]

3.0	Study Goals and Objectives

The overall goal of this three-phased study is to determine the need for and appropriate configuration of a Narrow 2 Powerhouse Intake modification.  The goal of the first phase of the study is to determine appropriate target water temperatures (i.e., temperature range, times of year and locations) for steelhead (Oncorhynchus mykiss irideus) and spring- and fall-run Chinook salmon (O. tshawytscha) in the Yuba River downstream of Englebright Dam.

The goal of the second phase of the study is to determine if the current Narrows 2 Powerhouse Intake, in conjunction with operation of other existing Project facilities can provide the target water temperatures and, if not, how the intake might be modified to provide the target temperatures.

The goal of the third phase is to develop a conceptual design for a Narrows 2 Powerhouse intake modification, if the second phase determines a modification to the intake is needed to meet target water temperatures.

The objective of the study is to develop information to meet the study goals as described above.

4.0	Existing Information and Need for Additional Information

4.1		History Temperature Regime in the Yuba River

The Yuba River downstream of Englebright Dam supports significant, naturally spawning populations of anadromous fish, including Central Valley fall-run Chinook salmon, spring-run Chinook salmon, and steelhead.  Central Valley spring-run Chinook salmon is currently listed as a threatened species under both the federal and State of California Endangered Species Acts (ESA and CESA, respectively), and steelhead is listed as threatened under the ESA. Central Valley fall-run Chinook salmon have been designated a candidate for listing under the federal ESA.

Since 1941, water temperature in the Yuba River downstream of Englebright Dam has been significantly affected by releases from the 260-foot-high Englebright Dam.  From 1941 through 1970, outflow from Englebright Reservoir was controlled through Pacific Gas and Electric Company’s (PG&E) Narrows 1 Project, constructed concurrently with Englebright Dam.  The Narrows 1 Powerhouse is located on the south bank of the Yuba River about 1,500 feet downstream of Englebright Dam, and the powerhouse has a maximum flow capacity of about 740 cfs.  Since Englebright Dam does not contain a low-level outlet, flows that could not be controlled by PG&E’s Narrows 1 Project overflowed the dam.  Englebright Reservoir is fairly small (70,000 acre-feet, or ac-ft, of total storage in 1970), and is reportedly over 25 percent filled with sediment. There was likely minimal cold water during the summer in Englebright Reservoir prior to the construction of New Bullards Bar Dam, and likely water temperatures in the Yuba River downstream of Englebright Dam were similar to natural flow conditions (i.e. quite warm in summer and fall periods).

In 1970, Licensee constructed the Yuba River Development Project, which included two significant components that lowered summertime water temperatures in the Yuba River downstream of Englebright Dam.  The first was the combination of New Bullards Bar Dam and New Colgate Powerhouse.  The dam forms New Bullards Bar Reservoir, which has a large supply of cold water.  That cold water is released to the Yuba River through New Colgate Powerhouse located just upstream of Englebright Reservoir.

The second Project component was Narrows 2 Powerhouse, which greatly increased the capability for controlled releases from Englebright Dam.  The intake for the Narrows 2 Powerhouse is a tower located on the northwest side of Englebright Reservoir adjacent to Englebright Dam.  The tower receives water from the surface of Englebright Reservoir down to an elevation of 439 feet, about 80 feet below the normal maximum water surface elevation.

As a result of the above, water temperature in the Yuba River downstream of Englebright Dam were significantly cooler in the late spring, summer and fall after 1970 than it had been from 1941 through 1970 and also cooler than pre-dam, natural conditions for much of the warmer months of the year.  However, water temperature continued to be a major concern of resource agencies for management and recovery of chinook salmon and steelhead on the Yuba River downstream of Englebright Dam (CDFG 1991).  To address this concern, starting in about 1999 YCWA conducted preliminary investigations of the potential benefits of installing an intake extension for the Narrows 2 Powerhouse that would access the colder, deeper water in Englebright Reservoir.  Initial modeling indicated that temperature reductions of 1 to 6 degrees Farenheit (°F) could potentially be achieved, depending on hydrologic conditions, release rates from New Bullards Bar Reservoir, and the final design and operation of a new intake.  YCWA pursued grant funding from CalFed and other sources for design and installation of an intake device; however, funding was not secured.  In 2000, the State Water Resources Control Board (SWRCB) in Order D-1644 directed YCWA to “diligently purse” funding for the intake extension.

Since the 2006, water temperatures in the Yuba River downstream of Englebright Dam have been lowered as a result of higher instream flows when the first Pilot Program year of the Yuba Accord was implemented.  The Lower Yuba Accord is a consensus‐based, comprehensive set of agreements designed, among other things, to protect and enhance the Yuba River downstream of Englebright Dam.  The EIR/EIS undertaken for the Accord demonstrated that summer and fall month temperatures in the Yuba River downstream of Englebright Dam would generally be colder by 1° to 5°F (depending on hydrologic conditions & release schedule) as a result of the Accord.  The EIR/EIS also demonstrated that the long-term average water temperatures in most year classes, including Dry and Critical water years would be lower than the CDFG 1991 suggested temperatures that initially prompted the consideration of a new intake structure for Narrows 2.

At this time, it appears appropriate to re-evaluate the potential Narrows 2 Powerhouse Intake extension in context with the FERC relicensing studies and Lower Yuba Accord River Management Team (RTM) studies.

4.2	Existing and Available Information for Temperature Regime in the Yuba River

Temperature conditions in the Yuba River below Englebright Dam have been evaluated on several occasions using a variety of techniques.  CDFG undertook an analysis using existing data in CDFG’s “Lower Yuba River Fisheries Management Plan” (CDFG 1991).  Additional analysis of temperature data was completed in support of the SWRCB hearings on flows for the Yuba River below Englebright (SWRI et al. 2000).

A Technical Team comprised of representatives from YCWA, NMFS, CDFG, USFWS, and non-governmental organizations undertook a qualitative review of temperature conditions in 2001-2003 during the course of the development of the Lower Yuba River Accord flow schedules.  (Yuba Accord Fisheries Agreement, 2005).  Subsequent analysis of the Accord flow schedules during the preparation of the Draft Environmental Impact Report/Environmental Impact Statement for the Proposed Lower Yuba River Accord (YCWA 2007) utilized additional temperature data and a temperature model developed for the EIR/EIS analysis to compare temperature conditions under various CEQA and NEPA scenarios including No Action/No Project and various flow alternatives.

Currently, the Lower Yuba RMT comprised of representatives from YCWA, NMFS, CDFG, USFWS, PG&E, and a group of NGO’s is revisiting temperature management considerations for the Yuba River below Englebright Dam, including species of concern and life stage periodicities, and preferred and acceptable water temperature ranges by species and life stage.  The RMT anticipates developing a water temperature target matrix during 2010, by location and month, for the Yuba River downstream of Englebright Dam that will balance the needs of various species and life stages in the river. Target water temperatures ranges may be developed for different locations in river, different periods of the year, and different life stages.  The RMT’s work will be summarized in a white paper report that will cite purpose, information sources, analysis and conclusions.  The RMT anticipates completing this task prior to May of 2011.

Since the RMT membership includes or represents all of the jurisdictional agencies and most concerned stakeholders for the Yuba River below Englebright Dam, it is anticipated that the RMT’s technical analysis of water temperature requirements the Yuba River below Englebright Dam will be sufficient for use by relicensing stakeholders, resource agencies, and YCWA for assessment of temperature impacts for the Lower Yuba River.

Relicensing stakeholders with an interest in issues related to temperatures in the Yuba River below Englebright Dam are encouraged to interact with their respective RMT representatives during the technical analysis.  Additionally, the RMT can make interim work products, data and findings available to the Relicensing Stakeholders

5.0	Study Methods and Analysis

5.1		Study Area

For the purpose of this study, the study area includes Englebright Dam in the vicinity of the Narrows 2 Powerhouse Intake and the Yuba River downstream of Englebright Dam.

This study will draw on information being developed primarily by two other Relicensing studies, each of which has a broader study area: Licensee’s Water Balance/Operations Model and Water Temperature Model Study.

If YCWA proposes an addition to the Project, the study area will be expanded if necessary to include areas potentially affected by the addition.

5.2		General Concepts

The following general concepts apply to the study:

- Personal safety is the most important consideration of each fieldwork team.  If YCWA determines the information cannot be collected in a safe manner, YCWA will develop other safe methods to collect the data.

- YCWA will make a good faith effort to obtain permission to access private property where needed well in advance of entering the property.

- The schedule for the study is reasonably flexible to accommodate unforeseen problems that may affect the schedule.

- Field crews may make minor modifications to the study proposal in the field to accommodate actual field conditions and unforeseen problems.  When modifications are made, YCWA field crew will follow the protocols in this study proposal.

- YCWA’s performance of the study does not presume that YCWA is responsible in whole or in part for measures that may arise from the study.

- Global Positioning System (GPS) data will be collected in a manner that meets or exceeds the United States Geological Survey’s (USGS) “National Map Accuracy Standards” for published maps.  All GPS data will be in the Universal Transverse Mercator (UTM) Coordinate System, using the North American Datum 1983 and stored in Environmental Science Research Institute (ESRI) Shapefile format.

5.3		Methods

The study will be conducted in three steps, each of which is described below.

5.3.1	Step 1 – Assess Ability of Existing Intake and Alternatives to Meet Target Water Temperatures

In this step, Licensee will use the Relicensing Water Balance/Operations Model in combination with the Relicensing Water Temperature Model to determine if, and under what conditions, the existing Narrows 2 Power Tunnel Intake structure can be used to meet the target water temperatures developed by the RMT.  The model will also be used to investigate if the withdrawal of water from deeper portions of the Englebright Reservoir would facilitate meeting the target water temperatures.  Licensee will consult with the Relicensing Participants regarding model assumptions, and model results.

If this step determines that the existing Narrow 2 Power Tunnel Intake as configured is adequate to meet target water temperatures, the study will skip Step 2 and proceed to Step 3.

5.3.2	Step 2 – Develop Conceptual Design for Preferred Alternative

If Step 1 determines that a reconfiguration of the Narrow 2 Power Tunnel Intake would be necessary to achieve target water temperatures, Licensee will develop alternative conceptual designs for intake.  The goal of this step is to develop enough detail so that the intake, including its construction, can be included in Licensee’s Application for a New Licensee and evaluated in the National Environmental Policy Act (NEPA) and California Environmental Quality Act (CEQA) reviews to support issuance of a new license by FERC, and all necessary agency permits and approvals to construct and operate the modified intake, if one is proposed.  Licensee will work collaboratively with the Relicensing Participants regarding the conceptual design.

5.3.3	Step 3 – Prepare Report

Licensee will prepare a report that includes the following sections: 1) Study Goals and Objectives; 2) Methods and Analysis; 3) Results; 4) Discussion; and 5) Description of Variances from the FERC-approved study proposal, if any.  The report will include the RMT’s final white paper, results of the model runs, and conceptual design and supporting information for the modified Narrows 2 Power Tunnel Intake, if one is proposed by Licensee.

6.0		Study-Specific Consultation

The study includes the following study-specific consultation:

The Licensee will consult with the Relicensing Participants regarding model assumptions, and model results during Step 1.

The Licensee will seek comments from the Relicensing Participants regarding the conceptual design.

7.0	Schedule

Licensee anticipates the schedule to complete the study as follows assuming the PAD is filed on November 1, 2010, and FERC issues its Study Determination by October 4, 2011:

Assess Existing and Alternative Intakes (Step 1)	November 2012 – March 2013

Develop Conceptual Design (Step 2)	April 2013 – July 2013

Prepare Report (Step 3)	August – October 2013

8.0		Consistency of Methodology with Generally Accepted Scientific Practices

The study is unlike studies performed for other relicensings, and therefore, does not conform to standard scientific practices.  However, identifying goals, assessing the ability of existing structures to meet those goals and developing conceptual designs for modifications to existing structures so the goals can be met meets the criteria for good engineering practice.

9.0	Level of Effort and Cost

[Relicensing Participants – Licensee will include a cost range estimate for this study in its Proposed Study Plan.  Licensee]

10.0	References Cited

California Department of Fish and Game, 1991.  Lower Yuba River Fisheries Management Plan. Stream Evaluation Report No. 91-1. Sacramento, CA.

Childs, J.R., Snyder, N.P., and Hampton, M.A., 2003, Bathymetric and geophysical surveys of Englebright Lake, Yuba-Nevada Counties, California, U.S. Geological Survey Open-File Report 03-383 http://geopubs.wr.usgs.gov/open-file/of03-383.

Yuba County Water Agency, California Department of Fish and Game, South Yuba River Citizens League, The Bay Institute, Trout Unlimited, Friends of the River, 2005. Lower Yuba River Fisheries Agreement  Sacramento, 2005.

Yuba Accord River Management Team. 2009. Lower Yuba River Accord Monitoring and Evaluation Program. Draft. June 2009. <www.yubaaccordrmt.com>

Yuba County Water Agency (YCWA). 2007. Draft Environmental Impact Report/Environmental Impact Statement for the Proposed Lower Yuba River Accord.  Prepared for the Department of Water Resources, Bureau of Reclamation and Yuba County Water Agency by HDR|SWRI.  June 2007.

Yuba County Water Agency (YCWA). 1998. Narrows II Powerhouse Intake Extension.  Project Proposal, Prepared for Yuba County Water Agency by Bookman-Edmonston Engineering.  October 1998.

Englebright Dam was constructed by the California Debris Commission in 1941; is owned, operated and maintained by the United States Army Corps of Engineers; and is not included as a Project facility in FERC licenses for the Yuba River Development Project.

Since the California Debris Commission designed Englebright Dam as a debris storage dam, it did not include a low-level outlet in the dam.

Under current operations, Englebright Reservoir fluctuates about 7 feet, from full pool at elevation 527 ft down to an elevation of about 520 feet, except during the winter and spring when the reservoir often spills over the Englebright Dam spillway.